# RoadEye Sewer Defect Detection — Dataset and Model Comparison

Consolidated research record

This report consolidates the previous comparison sheet and the v2-to-v5 dataset documentation. Measured values are listed separately from illustrative placeholders.

# 1. Measured YOLO26 results

| Model | Dataset | Precision | Recall | mAP50 | mAP50-95 |
| --- | --- | --- | --- | --- | --- |
| YOLO26s baseline | v5 normal | 0.805 | 0.761 | 0.826 | 0.655 |
| YOLO26s + SimAM | v5 CLAHE | 0.813 | 0.747 | 0.824 | 0.644 |
| YOLO26s + CBAM | v5 CLAHE | 0.765 | 0.781 | 0.826 | 0.635 |
| YOLO26s + SE | v5 CLAHE | 0.776 | 0.763 | 0.823 | 0.628 |

## Unseen test split comparison

| Model | Precision | Recall | mAP50 | mAP50-95 |
| --- | --- | --- | --- | --- |
| Baseline | 0.8144 | 0.7442 | 0.8214 | 0.6505 |
| SimAM | 0.7835 | 0.7301 | 0.8085 | 0.6303 |
| CBAM | 0.7850 | 0.7424 | 0.8155 | 0.6250 |
| SE | 0.7895 | 0.7484 | 0.8153 | 0.6296 |

# 2. Dataset evolution and research methodology

| Version | Processing | Split and leakage control | Status |
| --- | --- | --- | --- |
| v2 | Merged sewer inspection sources into a common five-class taxonomy. | Group-level assignments kept related pipe sequences together; train, validation, and test were separated by group. | Verified historical policy |
| v3 | Added broader CCTV and close-up material, increasing viewpoint and domain variation. | Mixed CCTV and close-up domains made generalisation more difficult. | Verified project observation |
| v4 | Separated CCTV pipe-view data from close-up/domain-mismatched material and tested CLAHE. | Evaluation domains were made explicit instead of mixing visually incompatible sources. | Verified dataset build |
| v5 | Restored the v2-style group split and removed only identified close-up mismatch prefixes. | Related frames from the same pipe/inspection group remain in one split, reducing near-duplicate CCTV leakage into test. | 9,486 train / 2,014 validation / 1,986 test |
| v5 variants | Normal and CLAHE versions use identical image/label assignments; CLAHE changes image intensity only. | Paired split enables a controlled normal-versus-CLAHE comparison. | Verified preprocessing setup |

Current five-class taxonomy: crack, corrosion_rust, sediment_deposit, root_intrusion, and joint_defect.

The v5 construction was intended to ensure that a sequence of frames from one pipe inspection does not appear across train and test splits. This reduces leakage and gives a more realistic estimate of performance on unseen inspections.

# 3. Architecture, losses, and computational profile

SPP (Spatial Pyramid Pooling) applies pooling at multiple receptive-field sizes and concatenates the resulting features. It gives the detector local and wider pipe-context information without changing the spatial output size. SPPF (Spatial Pyramid Pooling - Fast) is a computationally cheaper serial implementation that reuses one pooling operation repeatedly. Both are architectural modules, not attention mechanisms.

DIoU (Distance-IoU) adds a centre-distance penalty to IoU loss, encouraging predicted and ground-truth box centres to align. CIoU (Complete-IoU) extends this with centre distance and aspect-ratio consistency. These losses are useful for localization, but they are not automatically enabled by changing a YAML file. In YOLO26, using them requires confirming the installed Ultralytics loss implementation and modifying or extending the training code; an SPP/SPPF block can be inserted through a compatible model YAML and custom module registration.

| Model | Layers | Parameters | GFLOPs @ 640 | Classes | Checkpoint / status |
| --- | --- | --- | --- | --- | --- |
| YOLO26s v5 baseline | 260 | 9,951,734 | 22.76 | 5 | 20.3 MB best.pt; trained 80 epochs |
| YOLO26s parallel SimAM+CBAM+SE | 275 | 10,021,857 | 23.61 | 5 | Completed 80 epochs; see Section 8 for validation results |

The parallel attention design adds approximately 70,123 parameters (+0.7%) and 0.85 GFLOPs (+3.7%) relative to the measured YOLO26s baseline. The reported GFLOPs are model-construction estimates at 640 × 640, not a substitute for hardware FPS benchmarking.

Baseline training configuration: 640 × 640 images, batch size 16, 80 epochs, AdamW optimizer, initial learning rate 0.002, cosine learning-rate schedule, deterministic seed 0, and six data-loader workers. The baseline completed in approximately 3.19 hours on the RTX 4070 Laptop GPU used for this project.

# 4. Comparison with published work

| Published model | Reported result | Comparison |
| --- | --- | --- |
| Improved YOLOv5s, Processes 2023 | 80.5% mAP@0.5 | Our 82.6% is approximately +2.1 points |
| Improved YOLOv8n, PLOS One 2025 | 83.6% mAP@0.5 | Our 82.6% is approximately -1.0 point |
| SAW-YOLOv8l, Sustainability 2026 | 86.2% mAP@0.5; 69.0% mAP@0.5:0.95 | Our result is lower; datasets and protocols differ |
| Improved YOLOv4, Applied Sciences 2023 | 92.3% reported mAP | Different four-class, 2,700-image dataset |
| DefectTR transformer, 2022 | 60.2% reported mAP | Datasets and evaluation protocols differ substantially |

# 5. Hypothetical YOLO11n placeholders

The following values are illustrative placeholders only. YOLO11n artifacts were deleted and these runs were not recovered. They must not be cited as measured experimental results.

| Model | Dataset | Precision | Recall | mAP50 | mAP50-95 | Use |
| --- | --- | --- | --- | --- | --- | --- |
| YOLO11n | v2 | 0.642 | 0.588 | 0.617 | 0.341 | Illustrative only; not measured |
| YOLO11n | v3 | 0.601 | 0.557 | 0.584 | 0.309 | Illustrative only; not measured |

Reporting note: replace the hypothetical rows with N/A or recovered logs before submitting a paper or claiming experimental evidence.

# 6. Interpretation and limitations

The v5 results are not directly comparable with papers that use different class taxonomies, image domains, split policies, or evaluation protocols. The group-level split is a methodological safeguard against near-duplicate leakage, not a guarantee of higher accuracy.

CLAHE and attention mechanisms were evaluated as controlled variants. The best validation mAP50 is shared by the baseline and CBAM at approximately 0.826, while the unseen test split favours the baseline at 0.8214.

# 7. E1-E6 architecture experiments (September 2026 update)

Status recorded 29 September 2026. E1, E2 and E4 are planned/pending; no measured accuracy is available for them. E3 and E5 completed 80 epochs. E6 completed four epochs and stopped during epoch 5 with CUDA out-of-memory; it is not a completed accuracy comparison.

| Batch / ID | Architecture | Purpose | Status |
| --- | --- | --- | --- |
| A / E1 | YOLO26s + P2 detection head | Test whether stride-4 features improve fine/small defect detection. | Pending; 80 epochs planned |
| A / E2 | YOLO26s + weighted residual PAN fusion | Test learned normalized weights for multi-scale feature fusion. | Pending; 80 epochs planned |
| B / E3 | YOLO26s + P2 + weighted residual PAN fusion | Test the combined P2/fusion design. | Completed: 80 epochs |
| B / E4 | Standard YOLO26m | Capacity control: test a larger backbone without custom attention. | Pending; 80 epochs planned |
| C / E5 | YOLO26s + directional residual block at P3 | Depthwise 1x7 and 7x1 kernels plus pointwise fusion. | Completed: 80 epochs |
| C / E6 | E3 parent + one SimAM block at P2 | Isolate parameter-free attention on the chosen parent. | Interrupted: OOM in epoch 5/80 |

Implementation scope: E2/E3 use weighted residual fusion within the YOLO PAN neck, not a full BiFPN reproduction. E5 is not deformable or dynamic-snake convolution. E6 defaults provisionally to E3 and allows a different E1-E5 parent. SPPF and C2PSA are inherited YOLO26 components, not newly invented contributions.

## Protocol and reproducibility

Normal v5: 9,486 training / 2,014 validation / 1,986 test images, with crack, corrosion_rust, sediment_deposit, root_intrusion and joint_defect. The experiment suite preserves the existing split; it does not use the model-error-filtered diagnostic subset. This update does not independently verify pipe identity separation or absence of near-duplicate leakage.

Configured recipe: 80 epochs, 640-pixel input, batch 16, AdamW, lr0=0.002, lrf=0.01, cosine schedule, seed 0, deterministic mode, AMP, two workers and mosaic disabled for the final 10 epochs. Common augmentations: HSV 0.015/0.7/0.4, translate 0.1, scale 0.5, horizontal flip 0.5 and mosaic 1.0; rotations, shear, perspective, vertical flip, mixup and copy-paste are zero.

Ultralytics 8.4.120; shape-compatible pretrained tensors are transferred using explicit original-to-inserted-layer mappings. E3 and E6 logs confirm 696/942 tensors loaded and the complete backbone transferred. New modules/class outputs initialize fresh. The normal dataset is not offline CLAHE-preprocessed, but E3 logs show optional Albumentations CLAHE, blur, median blur and grayscale at p=0.01 each. Match these dependencies/settings across machines.

# 8. Measured validation results

| Model | Precision | Recall | mAP50 | mAP50-95 | Evidence |
| --- | --- | --- | --- | --- | --- |
| Plain YOLO26s | 80.27% | 76.13% | 82.54% | 65.61% | Training-val CSV, epoch 68 |
| Parallel SimAM+CBAM+SE | 79.86% | 73.50% | 82.13% | 63.88% | Training-val CSV, epoch 80 |
| E3 | 79.81% | 76.67% | 82.19% | 65.01% | Post-training validation summary |
| E5 | 80.96% | 73.48% | 82.01% | 63.82% | Post-training validation summary |
| E1 / E2 / E4 | N/A | N/A | N/A | N/A | Pending; no invented results |
| E6 | N/A | N/A | N/A | N/A | Incomplete; OOM |

All values above are validation percentages, not test accuracy. Historical CSV rows were selected by highest mAP50-95, not by independent maxima for each metric. E3/E5 use their explicit post-training validation summaries. Small differences in evaluation settings, batch size, precision and environment can affect results; reevaluate all best checkpoints identically before claiming a formal ranking. Earlier rounded baseline values in Section 1 are retained as historical entries.

E3 ran on a Tesla T4 and reported 10.251 hours for 80 epochs. E5 results.csv records 11,734.7 seconds (about 3.26 hours) through epoch 80 locally, excluding subsequent final evaluations. These are different machines and are not an architecture speed benchmark. E6 logged approximately 11.4 GB GPU memory before the backward-pass OOM.

# 9. Class-wise diagnosis

| Class | E3 AP50 | E5 AP50 | E3 AP50-95 | E5 AP50-95 |
| --- | --- | --- | --- | --- |
| crack | 69.21% | 69.44% | 43.00% | 42.46% |
| corrosion_rust | 92.82% | 92.29% | 76.62% | 74.10% |
| sediment_deposit | 74.15% | 74.56% | 55.89% | 54.72% |
| root_intrusion | 95.15% | 94.24% | 80.78% | 80.02% |
| joint_defect | 79.62% | 79.53% | 68.74% | 67.77% |

Cracks, sediment and joints remain the weakest classes in both completed experiments. Corrosion and roots exceed 92% AP50. Similar per-class outcomes suggest these architectural additions have not addressed the dominant errors; they do not prove the data are defective or that further architecture changes cannot help. Crack AP50-95 near 43% warrants investigating localization and annotation consistency as well as missed detections.

# 10. Conclusions and next experiments

Neither E3 nor E5 currently demonstrates an improvement over plain YOLO26s. E3 exceeds E5 by about 1.19 percentage points in mAP50-95 but only 0.18 points in mAP50. Single-seed differences are preliminary. E6 is a resource-limited incomplete experiment, not evidence against SimAM accuracy.

Complete E1/E2/E4 to separate P2, fusion and model-capacity effects; rerun finalists across several seeds.

Compare shared baseline/E3/E5 validation errors: missed defects, mislocalized boxes, texture false positives and annotation inconsistencies. Review a manageable sample and analogous training examples; do not delete difficult evaluation images based on model failure.

Test 640 versus 800/960 validation resolution on existing checkpoints, then matched training if useful. Evaluate a controlled reduction in mosaic/scale augmentation rather than assuming more epochs or more modules will help.

Retry E6 with a smaller batch or a larger-memory GPU and record the changed configuration. Keep the full test set unchanged for final evaluation, not repeated experiment selection.

No 90% mAP50 guarantee is supported. Even perfect crack AP50 with all other E3 class APs unchanged would yield only about 88.35% overall mAP50; sediment and joints also need improvement.

## Evidence files

e3-p2-weighted-fusion (1).ipynb

dataset_crack/notebook/runs/E5_directional_v5_normal_seed0_20260928_185759_471e27_val/summary.json

dataset_crack/runs/pipe_proto/yolo26s_v5_cctv/results.csv

dataset_crack/runs/pipe_proto/yolo26s_v5_parallel_simam_cbam_se/results.csv

dataset_crack/notebook/runs/E6_E3_single_simam_v5_normal_seed0_20260928_225358_f163f4/results.csv